Date: 26/02/2023                                                              Tender #: HOPE/2023/006

Company Name:

Subject: Invitation to Bids (ITB) for Rehabilitation of Mini Water Yard (MWY) in Bindisi Janoub - Bindisi locality – Central Darfur State

الموضوع: دعوة لتقديم العطاء لصيانة محطة مياه صغيرة في بندسي جنوب - محلية بندسي - ولاية وسط دارفور

| No. | Description/ Specification | Unit | Quantity | Unit cost/ Price SDG | Total Cost/ Price SDG |
| --- | --- | --- | --- | --- | --- |
| 1 | Pumping test: |  |  |  |  |
| 1.1 | Conduct pumping test (constant and recovery test) to assess the well productivity discharge test including installation, removal of test pumping equipment, water level observations and draw down measurements. | job | 1 |  |  |
| 2 | Connection pipes: |  |  |  |  |
| 2.1 | Supply materials and Connect Distribution system using 2 inch HDPE pipe for the line from BH to elevated tank and distribution to the two tab stands and animal troughs with their required fittings as per drawings and instructed by the engineer. | ml | 65 |  |  |
| 3 | Human drinking water (tab stand) in school: |  |  |  |  |
| 3.1 | Supply and construct new tap stand at school with 6 taps (copper type 0.5 inch) and control valve and construct basin to drain excess water of (1.7 m * 3 m) with drain channel minimum 5m away from fencing. and connected by pipe line 2 inch HDPE from Elevated Tank to School. | PCS | 1 |  |  |
| 4 | Submersible pump: |  |  |  |  |
| 4.1 | Supply and install submersible pump 100 watt gronfouse 3D type 1.30 inch, 100 m head , Q = (7 - 10 ) M3 /Hr complete with all accessories ready for installation (rising pipes# 15 pcs 1,25 inch , cable copper type 4mm #5 role) according to pumping test result. | Job | 1 |  |  |
| 4.2 | Copper Taps of 1/3 inch for another existing tap stand | PCS | 12 |  |  |
| 4.3 | Cement for rehabilitation | Sack | 10 |  |  |
| 5 | Solar system / Generator: |  |  |  |  |
| 5.1 | Supply and install of solar system unit for pump complete with panel 280 watt capacity (GSB) type for each one and should be fixed above the tower of elevated tank, including solar stand from (UC) iron angel 2 inch | panel | 8 |  |  |
| 5.2 | Supply and install inverter SP 1800 /controller according to pumping test result and submersible pump capacity. | PCS | 1 |  |  |
| 6 | Open well maintenance: |  |  |  |  |
| 6.1 | Labor Cost (including Open well maintenance, installation of solar system, fencing maintenance, installation of pump with tank). | job | 1 |  |  |
|  | Total offer for Rehabilitation of MWY |  |  |  |  |

Below requirements should be submitted to HOPE Office at Zalingei: Deadline: 12 February 2023

- Fill the stamped Public Tender/ RFQ.

- Company Profile included relevant achievements.

- Copies for all company’s certificates.

The tender requirements will be submitted to HOPE office at Zalingei as below:

Address: House (205), Karanik Shamal, Zalingei Central Darfur State – Sudan\

منزل رقم (205) حي الكرانك شمال، زالنجي – ولاية وسط دارفور - السودان

Contact: 0120023565 – 0120023522 - 0120023561

Thank you.

Yours᾿ Sincerely,

Ms. Aisha Ahmed Hassan, Executive Director, HOPE