Date: 26/02/2023                                                              Tender #: HOPE/2023/005

Company Name:

Subject: Invitation to Bids (ITB) for Rehabilitation of Mini Water Yard (MWY) in Fasi - Zalingei locality – Central Darfur State

الموضوع: دعوة لتقديم العطاء لصيانة محطة مياه صغيرة في فاسي - محلية زالنجي - ولاية وسط دارفور

| No. | Description/ Specification | Unit | Quantity | Unit cost/ Price SDG | Total Cost/ Price SDG |
| --- | --- | --- | --- | --- | --- |
| 1 | Pumping test: |  |  |  |  |
| 1.1 | Conduct pumping test (constant and recovery test) to assess the well productivity discharge test including installation, removal of test pumping equipment, water level observations and draw down measurements. | job | 1 |  |  |
| 2 | Connection pipes: |  |  |  |  |
| 2.1 | Supply materials and Connect Distribution system using 2 inch HDPE pipe for the line from BH to elevated tank and distribution to the two tab stands. | ml | 350 |  |  |
| 2.2 | Supply materials and lay Connection pipes (galvanize heavy type) 2 inch for inlet of the tank with the required fittings . | ml | 9 |  |  |
| 3 | Human drinking water (tab stand): |  |  |  |  |
| 3.1 | Maintenance and rehabilitate of existing tap stand (# 1 tap stand ) by 12 copper taps 1 inch | PCS | 1 |  |  |
| 4 | Submersible pump: |  |  |  |  |
| 4.1 | Supply and install submersible pump 100 watt gronfouse 3D type 1.30 inch, 100 m head, Q = (7 - 10 ) M3 /Hr complete with all accessories ready for installation (rising pipes# 15 pcs 1,25 inch, cable copper type 4mm #5 role) according to pumping test result. | Job | 1 |  |  |
| 5 | Solar system / Generator: |  |  |  |  |
| 5.1 | Supply and install of solar system unit for pump complete with panel 280 watt capacity (GSB) type for each one and should be fixed above the tower of elevated tank, including solar stand from (UC) iron angel 2 inch | panel | 8 |  |  |
| 5.2 | Supply and install inverter SP 1800 /controller according to pumping test result and submersible pump capacity. | PCS | 1 |  |  |
| 5.3 | Back-up generator (Galdong make, 7.5KV dinamo, 18HP) | PCS | 1 |  |  |
| 5.4 | Generator room 3x3 m square built of Iron frame fitted with Iron sheet, with wire mesh ventilation. | PCS | 1 |  |  |
| 6 | Fencing 10*10 m2: |  |  |  |  |
| 6.1 | Supply and construct 30 cm thick red brick wall for outer fencing of 1.0m height, 40 m length use clay mortar with plastering of cement sand mortar mixing in 1:6 , and complete the fencing of 1m height of x band (heavy type) should be fixed above the wall and internal separation use square steel pipes 4*8 cm size for columns and 3*6cm for horizontal lines (heavy type) and install circular wire (sharp type) of 35cm diameter above the x band, for borehole area should be fenced of (3*3m2) by gabion wire of 4mm thick type, the water yard complete with 2 gates inside door is (1.0 width &2m height) for outside door (2m width & 2m height). | job | 1 |  |  |
| 7 | Flooring : area (10 x 10 ) m: |  |  |  |  |
| 7.1 | Supply and construct interlock with sand for flooring in the total area. | m2 | 100 |  |  |
|  | Total offer for Rehabilitation of MWY |  |  |  |  |

Below requirements should be submitted to HOPE Office at Zalingei: Deadline: 12 February 2023

- Fill the stamped Public Tender/ RFQ.

- Company Profile included relevant achievements.

- Copies for all company’s certificates.

The tender requirements will be submitted to HOPE office at Zalingei as below:

Address: House (205), Karanik Shamal, Zalingei Central Darfur State – Sudan\

منزل رقم (205) حي الكرانك شمال، زالنجي – ولاية وسط دارفور - السودان

Contact: 0120023565 – 0120023522 - 0120023561

Thank you.

Yours᾿ Sincerely,

Ms. Aisha Ahmed Hassan, Executive Director, HOPE