Description of the Establishment of Sales Points in Girba and Halfa Localities, Kassala State

The establishment of sales points in Girba locality (Ard El-Hagar and Dar El-Naeem) and Halfa locality (Village 4 and Village 5) aims to enhance the accessibility and distribution of products within Kassala State. These sales points are designed to strengthen the organization’s market presence, ensure efficient service delivery, and directly support revenue growth in line with the organization’s overall sales and operational strategies.

Each sales point will serve as a centralized facility providing customers with direct access to products and services, while also functioning as a local hub for customer engagement, information dissemination, and sales support.

## 2. Objectives

To construct safe and durable buildings for seed storage and preparation.

To provide appropriate environmental conditions (temperature, ventilation, and power supply) for seed preservation.

To integrate renewable energy solutions for sustainability.

To ensure compliance with international and local engineering and safety standards.

## 3. Scope of Works

Buildings:

2Room (3 × 3 m, 3 m height) as layout drawing.

Structures include strip foundations, reinforced concrete beams/slabs, red brick walls, plastering, painting, and tiled flooring in accordance with BS /ISO/ASTM standard

The contractor shall supply all labour ,materials .and all operation necessary to construct 300 mm thick and 3.5 high as per drawing and specification and instructions of the engineer ,work include foundations ,DPC,masonry ,reinforcement ,plastering,curing and final cleaning.

Damp-proof from polyethylene minimum 300-500 micron thickness  install under strip foundation (600*250)

Ensure masonry work level and plump tolerances :10 mm over 2 m.

Plaster & finishes on both faces smooth and Cure plaster for 7 days.

Finishing & Site Cleaning:

Internal and external painting (primer + 2 finishing coats).

Floor finishing with 50 × 50 cm borclin tiles.

Site cleaning and debris removal prior to handover.

## 4. Technical Specifications

All works shall comply with:

BS 8110 – Structural Use of Concrete

BS 4449 – Reinforcing Steel

BS 8004 – Foundations

BS 648 – Aggregates for Concrete

Sudanese Standards (SSMO) – applicable building codes and material specifications

Concrete Works:

Foundations: C25 concrete (compressive strength 25 N/mm²).

Slabs: C25 concrete (compressive strength 25 N/mm²).

Beams : C25 concrete (compressive strength 25N/mm²).

Water/cement ratio ≤ 0.5.

Minimum curing period: 7 days.

Reinforcement:

High-yield steel bars fy = 460 N/mm².

Concrete cover: 25 mm (slabs), 30 mm (beams), 50 mm (foundations).

Stirrups T8 .

Masonry Works:

Red brick, Class 1, cement-sand mortar (1:6).

Plaster: 10 mm thickness, smooth finish mortar(1:6)

Paint: 1 primer + 2 finishing coats.

Flooring:

Compacted hardcore 150 mm + sand blinding 50 mm + plain concrete 100 mm and Finished with 50 × 50 cm white borclin tiles with good drainage.

Doors and Windows:

Supply and installation high -Quality doors and windows,fabricated from durable materials(steel,UPVC,WOOD ,GLASS )as specified complete with factory made locking system.

All doors and windows ahall be properly aligned ,well-fitted and tighty sealed with the buliding structure to ensure durability,safety and wetther resistance.

Hinges,handle and other accessory shall be of Heavy-duty type,rust-resistance and conform to international guilty standards.

Roof:

Provide and construct a reinforcement concrete roof slab with supporting beams

Proper shuttering and centering shall be provided to ensure correct alignment .level and finish

Adequate curing shall be carried out for at least 7-14 days to achieve required strength.

Provide and apply a complete waterproofing system to the roof slab to prevent water leakage and ensure long-term protection.

Provide and install a complete rainwater drainage system for the reinforcement concrete roof slab by:

- Roof slab shall be cast with a proper slope (minimum 1-2%) to ensure smooth flow of rainwater towards drainage point.

- Install (PVC/UPVC) rainwater downpipe fixed to the structure

- Down pipes shall discharge rainwater away from the building foundation through proper outlets,channels .

## 5. Deliverables

The Contractor shall deliver:

Completed sales point buildings .

Finished floors, walls, and painted surfaces.

Cleaned site.

## 6. Quality Assurance

All works shall comply with BS ,ISO/ASTM and Sudanese Standards (SSMO).

All materials must be tested and approved prior to use.

The supervising engineer will inspect and approve each stage before continuation.

Any non-conforming works must be rectified at the contractor’s cost.

## 7. Material Specification Changes

Any modification or change in the type or specification of materials or components shall only be carried out with the prior written approval of the supervising engineer. Any materials supplied or installed without this approval shall be considered non-compliant and must be removed or replaced immediately at the contractor’s expense.

## 8. Bill of Quantities for one Unit:

| No # | Description | Quantity | Unit | Estimated Unit Price | Estimated Total Price |
| --- | --- | --- | --- | --- | --- |
| 1 | Excavation for Foundation (Min. 60 width X 60 cm Depth minimum) measured from the lower point at site. The soil should be removed and discarded away from the site as directed by the engineer | 21 | M.L. |  |  |
| 2 | Approved hard earth back filling under floor (50 cm height) free from vegetation or any other foreign matter deposited in layers of 250mm watered and well compacted, as directed by the engineer. | 9 | M3 |  |  |
| 3 | Provide & Cast plain Concrete 1:3:6 before strip beam 10cm height, as directed by the engineer. | 21 | M.L. |  |  |
| 4 | Provide & Cast Reinforced Concrete 1:2:4 Grade 25 on the strip beam 50cm width and 25cm height), reinforced with 6 Nos.16 mm bars with stirrups ø8 @20cm c/c, water proof . Rates to include for form work and curing for 7 days min., as directed by the engineer. | 2.6 | m3 |  |  |
| 5 | Provide & Cast Reinforced Concrete 1:2:4 ilntel beam (20x20cm) at lintel level with 4 Nos. 12 mm bars and stirrups of ø8 mm@ 20cm c/c, for the main building, as per drawings. Rates to include for form work and curing for 7 days min., as directed by the engineer. | 12 | M.L. |  |  |
| 6 | Provide & Cast Reinforced Concrete 1:2:4 Grade 25 roof slab with beam by 12 cm thickness) reinforced with 12mm bars 15cm c/c and concerning negative reinforcement at edges two ways bottom grid. Rates to include for form work, workmanship and curing for 7 days min., as directed by the engineer. | 18 | M² |  |  |
| 7 | Supply, prepare and apply on the top of roof of the building roofs water and moisture insulating layer “khafja” with 13cm average thickness layer, comprised of 1:2:5 limes: Sand: Bricks mix. The mix to be fermented and further mixed with cement in 1:5 ratio. 1:6 cement sand screed to be applied to the entire roof after applying and maturing of Khafja layer. In addition, 3 coats of bitumen-based coat to be applied to the whole roof, to the satisfaction of the Engineer | 18 | M² |  |  |
| 8 | 30 cm thick cement block masonry (Gassa ) set on 1:6 sand cement mortars (3 vertical courses above the S. Beam). Rate to include masons labor works and provision of red brick ,sand, cement and water and curing for 3 days minimum. | 21 | M.L. |  |  |
| 9 | 20 cm thick cement block masonry set on 1:6 sand cement mortar in the wall. Rate to include masons labor works and provision of sand, cement and water and curing for 3 days minimum, for the main building. | 63 | M² |  |  |
| 10 | D1 size (1200x2200) mm | 2 | No. |  |  |
| 11 | W1 size (1400x1200) mm | 2 | No. |  |  |
| 12 | Steel Window Grill: Solid steel frame and bars: Provide and fix grill to windows, sizes (1400*1200) mm with (40 x 40) mm flat bar frame, including two equal-spaced horizontal anti-pry bars. Vertical bars of 16mm diameter solid steel or square pipe sections 100mm apart. Price include two coats of oil paint, to the satisfaction of the Engineer. | 2 | No. |  |  |
| 13 | Supply & Apply Painting (under coat and water-based paint), internally & externally in the entire building, using premier quality materials. A sample should be submitted for approval prior to delivery at site. The color to be approved by the engineer. The rate to include all materials, abrasive, workmanship and all requirements and need to implement the job properly and to the directions of the engineer. | 126 | M² |  |  |
| 14 | Plaster Works: Supply, provide & apply Internal & external Plastering 1:8 mix finished semi rough for external walls & smooth internally, in the main building. The rate of square meter to include plastering of blusters, parapet and narrow widths - voids are not considered in the measuring- curing up to three days’ minimum. | 125 | M² |  |  |
| 15 | Porcelain Floor Tiles: Provide and fix (500x500x8) mm anti slip Chinese porcelain floor tiles, fixed on c/s screed 1:3 mix, hardness rating shall be 50%, water absorption shall not exceed 3%, at rooms and veranda. | 18 | M3 |  |  |
| 16 | Provide materials, prepare, paint, colour, and fix with concrete footings (30X30X60cm) steel sign post 120X80 cm, with the logos of practical action and its donors, adhering to instruction of Engineer and artisan professional standard. | 1 | Job |  |  |
|  | Total |  |  |  |  |

## 9. Handover

Upon completion, the contractor shall:

Hand over the facility in a clean and fully operational condition.