Technical Specifications for Outdoor Video-Podcast Production

Podcast Setup: 1 Interviewer, 1 guest

Location: Outdoor environment representing actual field conditions

Purpose: Demonstrate audio clarity, camera quality, lighting control, and overall production capability.

Objective:

The supplier is requested to provide a proposal demonstrating capacity for professional outdoor video and audio production. As part of the evaluation, a 50 -second outdoor sample must be produced featuring one producer and one guest, reflecting the conditions and quality expected for the actual interviews.

1. Camera & Visual Capture

The supplier must provide a multi-camera setup capable of 10-bit colour depth for professional post-production.

Main Cameras: Minimum 3x Sony 4K camcorders (or equivalent Cinema Line cameras) to ensure consistent colour profiles and high dynamic range.

Lenses: A range of lens options suitable for multi-camera outdoor production, allowing flexibility based on the role of each camera (wide, medium, or close-up shots). The supplier should propose appropriate lenses (prime and/or zoom) to ensure visual consistency, depth of field control, and adaptability to changing outdoor conditions.

Support: 4x professional fluid-head tripods stable in windy outdoor conditions.

Monitoring: 1x 7-inch director’s monitor (high brightness for daylight visibility) and a multi-view switcher for on-site review.

Video Mixer: 1x ATEM Mini Pro (or similar) for “live-to-tape” switching to provide the client with an on-site rough cut.

2. Audio Recording

Given the podcast format, high-quality audio is essential, especially outdoors.

Microphone Suite:

Livelier Mics: 4x wireless sets (e.g., Sennheiser G4 or Sony UWP) with “deadcat” windscreens.

Shotgun Mics: 2x boom mics (e.g., Rode NTG3 or Sennheiser MKH416) to capture ambient sound and backup dialogue.

Dynamic Podcast Mics: 4x Shure SM7B or MV7 with heavy-duty stands for an on-camera podcast look.

Field Recorder/Mixer: 1x multi-track recorder (e.g., Zoom F6 or Sound Devices MixPre) to capture each participant on separate tracks.

Sound Treatment: Portable acoustic blankets and C-stands to reduce wind noise or echoes.

3. Lighting & Power

Filming outdoors in Sudan requires management of harsh sunlight and varying lighting conditions.

Key Lights: 3x high-output COB LED lights (e.g., Aperture 300d/600d) with V-mount battery compatibility.

Modifiers: Large soft boxes with grids and 5-in-5 diffusers to soften sunlight on subjects.

Grip & Safety: Sandbags, C-stands, and clamps for secure equipment placement.

Power Backup: Silent standby generator for locations without electricity.

4. Mobile Studio & Logistics

Portable Set: Collapsible, visually appealing outdoor studio kit (chairs, tables).

Transportation: The supplier is responsible for providing a dedicated 4x4 vehicle to safely transport the crew and all equipment across all locations, including inter-state travel.

5. Personnel Requirements

The vendor must propose a qualified team with field production experience:

1x Production Manager – overall team management

1x Director – quality control and artistic oversight

1x Sound Engineer – specialist in outdoor audio capture

3x Camera Operators – experienced with gimbal and tripod setups

2x Production Assistants – logistics and equipment management

1x Video Editor – post-production and editing

Sample Requirement:

As part of the vendor evaluation, a 50-second outdoor video sample must be produced: